ADSR Envelope Verification Test Results:

Test 1 – ADSR Envelope Timing Verification

Results

The ADSR envelope waveform was captured using an oscilloscope, allowing accurate measurement of the timing characteristics associated with each stage of the envelope. The results obtained were compared directly with the target design specifications to assess performance:

| Parameter | Target Value | Measured Value |
| --- | --- | --- |
| Attack Time | 100 ms | 105 ms |
| Decay Time | 200 ms | 210 ms |
| Release Time | 300 ms | 295 ms |

All measured values fall within the ±10% tolerance range specified in the verification protocol. The waveform observed on the oscilloscope displayed the expected exponential charging and discharging behaviour typical of an ADSR envelope generator.

Explanation

The results demonstrate that the timing performance of the ADSR envelope generator is closely aligned with the intended design. The small variations between the target and measured values are expected in analogue circuits and are primarily due to component tolerances. Resistors and capacitors used in the timing network typically have tolerances of ±5% or higher, which directly influence the RC time constants that determine the attack, decay, and release durations.

The slightly longer attack and decay times suggest that the effective resistance or capacitance in those stages may be marginally higher than their nominal values. This could be due to manufacturing tolerances or minor variations in the power supply conditions. The release time, being slightly shorter than expected, may result from similar effects but in the opposite direction.

In addition, environmental factors such as temperature and supply voltage fluctuations can influence analogue circuit behaviour. Despite these factors, the consistency of the results across repeated measurements indicates that the circuit is stable and reliable. The waveform shape remained consistent throughout testing, with no irregularities or distortion observed.

Conclusion

The ADSR envelope generator successfully meets the timing requirements defined in the system specifications. All measured parameters fall within the acceptable tolerance range, confirming that the circuit performs as expected.

The minor deviations observed are typical of analogue systems and do not significantly affect functionality. The circuit demonstrates reliable and repeatable behaviour, making it suitable for use in the synthesiser system. Overall, this test verifies that the envelope generator can accurately control the timing of signal amplitude, which is essential for shaping sound in audio applications.