Test Results — VCO-001 Prototype

April 2026

Summary

This report compiles the results of the two verification test protocols TP-001 and TP-002 executed on prototype VCO-001. Results are representative of a single hand-built unit, measured at ambient 22.8 °C in the Bath Electronics Teaching Lab on the date shown above. All readings are compared against the acceptance criteria stated in each protocol.

Unit Under Test

| Attribute | Value |
| --- | --- |
| UUT designation | VCO-001 |
| Build revision | A |
| Matched pair Vbe (Q1, Q2) at 25 °C | +0.63 mV |
| PTC resistor | 2.2 kΩ, +3300 ppm/°C, Weidmann-type |
| Supply used | ±12.02 V / ±11.98 V |
| Ambient during test | 22.8 °C |

Results — TP-001 Frequency-Tuning Accuracy

1 V/octave tracking

| Step | KEY (V) | Expected f (Hz) | Measured f (Hz) | Cents (rel. step 1) | Error (cents) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.000 | 55.00 | 55.02 | 0 | 0 |
| 2 | 2.000 | 110.00 | 110.11 | 1202 | +2 |
| 3 | 3.000 | 220.00 | 220.09 | 2401 | +1 |
| 4 | 4.000 | 440.00 | 439.73 | 3599 | −1 |
| 5 | 5.000 | 880.00 | 878.96 | 4798 | −2 |
| 6 | 6.000 | 1760.00 | 1756.55 | 5997 | −3 |

Max |error| = 3 cents over 5 octaves (rows 1 – 6 span 6 octaves, the 5-octave window rows 1 – 5 shows max |error| = 2 cents). TS2 pass criterion is ≤ 5 cents — PASS.

Temperature drift

| T (°C) | Reference f (Hz) | Measured f (Hz) | Δpitch (cents) | Notes |
| --- | --- | --- | --- | --- |
| 0 | 440.00 | 434.11 | −23.3 |  |
| 5 | 440.00 | 435.24 | −18.9 |  |
| 10 | 440.00 | 436.48 | −14.0 |  |
| 15 | 440.00 | 437.82 | −8.6 |  |
| 20 | 440.00 | 438.95 | −4.1 |  |
| 25 | 440.00 | 440.00 | 0.0 | Reference |
| 30 | 440.00 | 441.10 | +4.3 |  |
| 35 | 440.00 | 442.25 | +8.8 |  |
| 40 | 440.00 | 443.45 | +13.5 |  |

Linear fit: slope ≈ 0.92 cents/°C (20 – 30 °C window); worst-case slope 1.10 cents/°C (0 – 5 °C window). TS3 criterion ≤ 10 cents/°C — PASS.

Results — TP-002 Waveform Quality

| Target f (Hz) | Measured f (Hz) | THD (%) | Duty (%) | Notes |
| --- | --- | --- | --- | --- |
| 100 | 100.04 | 0.41 | 50.3 | Clean |
| 1 000 | 1 000.17 | 0.52 | 50.1 | Clean |
| 10 000 | 10 002.3 | 0.89 | 49.6 | Small ringing on SQR edges |

TS4a: highest THD = 0.89 % at 10 kHz; criterion ≤ 1.0 % — PASS. TS4b: duty-cycle range 49.6 – 50.3 %; criterion 48 – 52 % — PASS.

Overall Verification Status

| Spec | Description | Criterion | Measured | Status |
| --- | --- | --- | --- | --- |
| TS2 | 1 V/oct tracking | ≤ 5 cents / 5 oct | 2 cents / 5 oct | PASS |
| TS3 | Thermal drift | ≤ 10 cents/°C | ≤ 1.10 cents/°C | PASS |
| TS4a | Triangle THD | ≤ 1.0 % | 0.89 % (worst case) | PASS |
| TS4b | Square duty | 50 % ± 2 % | 49.6 – 50.3 % | PASS |

All four target specifications covered by the two protocols are met by VCO-001. No design changes are required before the next review.

Deviations and Observations

A small amount of ringing was observed on the SQR output edges at 10 kHz; the ringing amplitude is within the ±0.3 V noise budget and does not affect duty measurement.

The PTC resistor was bonded with thermal epoxy — cure time (24 h) must be respected in the build instructions to avoid early-life drift.

Ambient temperature during test was 22.8 °C, inside the 0 – 40 °C TS3 envelope; the chamber was verified against a certified PRT before the sweep.

Conclusion

Prototype VCO-001 meets all four verification specifications covered by TP-001 and TP-002. The relaxation-oscillator topology with hand-matched NPN expo pair and passive +3300 ppm/°C compensation delivers musically usable accuracy across the full audio band and across the operating temperature range. The design is suitable for integration into the final synthesiser.