jm3767, eckl21, jf2280, kce27, sps80, yll201

Date: 15/03/26

ADSR Envelope Timing Verification Test Protocol

# 1. Scope

This protocol verifies that the ADSR envelope generator produces timing characteristics that meet the design specifications. The test measures the duration of each envelope stage - Attack, Decay, Sustain, and Release - to confirm the circuit operates within the acceptable tolerance limits defined in the system specifications.

# 2. Specifications Under Test

| Ref | Target Specification | Pass Criterion |
| --- | --- | --- |
| TS1 | Attack time = 100 ms +/- 10% | Measured attack between 90 ms and 110 ms |
| TS2 | Decay time = 200 ms +/- 10% | Measured decay between 180 ms and 220 ms |
| TS3 | Release time = 300 ms +/- 10% | Measured release between 270 ms and 330 ms |
| TS4 | Sustain voltage stable while gate is held high | Voltage variation < +/-5% of set sustain level |

# 3. Equipment Required

| Ref | Item | Required Accuracy | Source |
| --- | --- | --- | --- |
| E1 | Digital oscilloscope (Tektronix TBS2104B or equivalent) | +/-0.5 ms timing resolution, 100 MHz BW | University lab |
| E2 | Gate trigger signal source (function generator) | 0-5 V logic level | University lab |
| E3 | ADSR envelope generator circuit | - | Project prototype |
| E4 | Regulated DC power supply (+/-12 V) | +/-0.05% voltage regulation | University lab |
| E5 | Oscilloscope probes (x10 passive) | 100 MHz BW | University lab |

# 4. Test Set-up

- Connect the ADSR circuit to the regulated DC power supply (E4) and verify the supply rails at +12 V and -12 V before proceeding.

- Connect the oscilloscope probe (E5) to the envelope output of the circuit. Use the x10 probe setting with the ground clip connected to the circuit ground.

- Connect the gate trigger source (E2) to the GATE input of the ADSR circuit. Configure it to produce a 5 V logic-level square wave with sufficient on-time to capture the full envelope.

- Configure the oscilloscope (E1) to capture the full ADSR envelope waveform. Set the time base to show at least 800 ms and the voltage range to show 0-5 V clearly.

- Set the oscilloscope trigger to the rising edge of the gate signal so the full envelope cycle is consistently captured on each trigger.

# 5. Test Procedure

- Apply a gate signal from E2 to trigger the envelope generator. Hold the gate high for at least 500 ms to ensure the sustain stage is clearly visible on the oscilloscope.

- Observe the waveform on E1. Confirm the expected exponential attack, decay, and release transitions are visible and free from artefacts or clipping.

- Measure the attack time: the elapsed time from the onset of the gate (0 V) to the point where the envelope reaches its peak voltage.

- Measure the decay time: the elapsed time from the peak voltage to the stable sustain voltage level.

- While the gate remains high, observe the sustain voltage over a 200 ms window and note any variation from the stable level.

- Remove the gate signal and measure the release time: the elapsed time from gate removal until the envelope output returns to 0 V.

- Repeat the full measurement sequence three times. Record all values and calculate the mean for each stage.

# 6. Acceptance Criteria

All four specifications must be satisfied for the circuit to pass this test. A failure on any single criterion requires root-cause analysis before the design may proceed.

| Ref | Criterion | Outcome if Not Met |
| --- | --- | --- |
| TS1 | Measured attack time within +/-10% of 100 ms target | FAIL - root-cause analysis required |
| TS2 | Measured decay time within +/-10% of 200 ms target | FAIL - root-cause analysis required |
| TS3 | Measured release time within +/-10% of 300 ms target | FAIL - root-cause analysis required |
| TS4 | Sustain voltage variation < +/-5% during gate-hold period | FAIL - root-cause analysis required |