VCF Cut-off Frequency Verification Test Protocol

Date: 15/03/26

1. Purpose

The purpose of this test is to verify that the Voltage Controlled Filter (VCF) achieves a controllable cut-off frequency within the specified range of 100 Hz to 5 kHz ±10%, as defined in the Design and Development (D&D) matrix.

2. Test Objectives

This test verifies the following requirements:

- N1.1 – The synthesiser must shape the harmonic content of the VCO output.

- S1.1 – Cut-off frequency adjustable from 100 Hz to 5 kHz ±10%.

- S1.2 – The filter must operate over at least three octaves of the audible range.

3. Equipment Required

- LTspice simulation environment

- VCF LTspice circuit file

- Square wave input source (±5 V)

- LTspice waveform viewer

4. Test Setup

- The VCF circuit is loaded into LTspice.

- A square wave input signal of approximately ±5 V is applied to the input of the filter.

- The output node is connected to the LTspice waveform viewer.

- The circuit is powered using the specified supply rails (e.g. ±9 V or ±12 V).

5. Test Procedure

- Set the cut-off control to its minimum position and run the simulation.

- Perform an AC sweep and determine the -3 dB cut-off frequency.

- Record the measured value.

- Repeat the procedure for the mid position of the control.

- Repeat again for the maximum position of the control.

6. Acceptance Criteria

- The minimum cut-off frequency must be at least 100 Hz within ±10%.

- The maximum cut-off frequency must reach at least 5 kHz within ±10%.

- The filter must demonstrate a usable range covering at least three octaves.

8. Verification Method

AC analysis in LTspice is used to determine the -3 dB cut-off frequency.

10. Notes

- Ensure that the input amplitude remains constant for all tests.

- Use a logarithmic frequency scale when analysing the response.