VCF Resonance Verification Test Protocol

Date: 15/03/26

1. Purpose

The purpose of this test is to verify that the VCF provides adjustable resonance and produces a measurable peak near the cut-off frequency in accordance with the D&D specification.

2. Test Objective

This test verifies the following requirements:

- N3 – The filter should provide resonance to make the sound more characterful.

- S3 – A peak magnitude of at least +3 dB ±1 dB near the cut-off frequency.

3. Equipment Required

- LTspice simulation environment

- VCF LTspice circuit file

- AC analysis tool in LTspice

4. Test Setup

- The VCF circuit is configured in LTspice.

- An AC sweep input signal is applied to the filter.

- The output is monitored using the frequency response plot.

5. Test Procedure

- Set the resonance control to its minimum position and perform an AC sweep.

- Record the baseline frequency response.

- Increase the resonance control and repeat the AC sweep.

- Observe and measure the peak gain near the cut-off frequency.

- Repeat the process for maximum resonance.

6. Acceptance Criteria

- The resonance peak must reach at least +3 dB within a tolerance of ±1 dB.

- The resonance must be smoothly adjustable using the control.

- The system must remain stable without unwanted oscillation during normal operation.

8. Verification Method

Frequency response analysis using LTspice AC sweep.

10. Notes

- Ensure consistent input signal conditions during all tests.

- Avoid excessive resonance settings that may introduce clipping or instability.